Mist NAC → FortiGate RADIUS Accounting Forwarder

Installation and Configuration Guide

Version 1.0

May 2026

# Table of Contents

# 1.  Overview

This guide covers the installation and configuration of the Mist NAC RADIUS Accounting Forwarder — a lightweight Python service that bridges Juniper Mist Network Access Control (NAC) events to a FortiGate firewall using RADIUS Accounting packets (RFC 2866).

## 1.1  How It Works

The authentication and identity flow is as follows:

- A wireless client authenticates to the network via EAP-TLS or EAP-PEAP through Mist NAC.

- Mist posts a webhook (HTTP POST) to this service for each accounting lifecycle event.

- The service builds a correctly-formed RFC-2866 RADIUS Accounting-Request packet and sends it to the FortiGate over UDP port 1813.

- FortiGate’s RADIUS SSO (RSSO) feature receives the packet and maps the username to the client IP address, establishing user identity for firewall policy enforcement.

## 1.2  Packet Types

Three Mist event types are handled and mapped to the corresponding RADIUS Acct-Status-Type values:

| Mist Event Type | RADIUS Status | Purpose |
| --- | --- | --- |
| NAC_ACCOUNTING_START | 1 – Start | User authenticated and joined the network |
| NAC_ACCOUNTING_UPDATE | 3 – Interim-Update | Periodic keepalive with traffic statistics |
| NAC_ACCOUNTING_STOP | 2 – Stop | User disconnected or de-authenticated |

## 1.3  RADIUS Attributes Sent

The following RADIUS AVPs are included in each packet type:

| Attribute | Code | START | UPDATE | STOP | Source Field |
| --- | --- | --- | --- | --- | --- |
| Acct-Status-Type | 40 | ✓ | ✓ | ✓ | Derived from event type |
| User-Name | 1 | ✓ | ✓ | ✓ | username |
| NAS-IP-Address | 4 | ✓ | ✓ | ✓ | nas_ip |
| Calling-Station-Id | 31 | ✓ | ✓ | ✓ | mac (XX-XX-XX-XX-XX-XX) |
| Acct-Session-Id | 44 | ✓ | ✓ | ✓ | session_id |
| NAS-Port-Type | 61 | ✓ | ✓ | ✓ | Fixed: 19 (Wireless 802.11) |
| Called-Station-Id | 30 | ✓ | ✓ | ✓ | ssid |
| Framed-IP-Address | 8 | – | ✓ | ✓ | client_ip |
| Acct-Input-Octets | 42 | – | ✓ | ✓ | rx_bytes |
| Acct-Output-Octets | 43 | – | ✓ | ✓ | tx_bytes |
| Acct-Input-Packets | 47 | – | ✓ | ✓ | rx_pkts |
| Acct-Output-Packets | 48 | – | ✓ | ✓ | tx_pkts |
| Acct-Session-Time | 46 | – | – | ✓ | session_duration_in_mins × 60 |
| Acct-Terminate-Cause | 49 | – | – | ✓ | terminate_cause |

Note: Mist does not include the client IP in the START event. FortiGate RSSO will use the Calling-Station-Id (MAC) for initial identity binding and add the IP when the first UPDATE or STOP is received.

# 2.  Prerequisites

## 2.1  Server Requirements

- Linux host (Ubuntu 22.04 / Debian 12 or later recommended)

- Python 3.8 or later — no third-party packages required

- Network access from the Mist cloud to this server on the configured webhook port (default 8080)

- Network access from this server to the FortiGate on UDP port 1813

## 2.2  Network Requirements

| Connection | Protocol / Port | Direction | Notes |
| --- | --- | --- | --- |
| Mist Cloud → This server | TCP/HTTP 8080 | Inbound | Port-forward through edge firewall |
| This server → FortiGate | UDP 1813 | Outbound | RADIUS Accounting (RFC 2866) |

## 2.3  Information Required Before Starting

Collect the following before proceeding:

- FortiGate management IP address (the interface that will receive RADIUS accounting)

- RADIUS shared secret — you will create this; it must match on both sides

- External IP address or hostname of this server (needed when configuring the Mist webhook)

- Mist Organisation ID (visible in the Mist dashboard URL)

# 3.  Installation

## 3.1  Copy the Script

Copy mist_nac_radius.py to a suitable location on your server:

sudo mkdir -p /opt/mist-radius

sudo cp mist_nac_radius.py /opt/mist-radius/

sudo chmod 755 /opt/mist-radius/mist_nac_radius.py

Create the log directory:

sudo mkdir -p /opt/mist-radius/logs

## 3.2  Verify Python Version

python3 --version

Output should show Python 3.8 or later. No pip install steps are required — the script uses only the Python standard library.

# 4.  Script Configuration

Open the script in a text editor:

sudo nano /opt/mist-radius/mist_nac_radius.py

All configuration is in the section at the top of the file marked “Configuration”.

## 4.1  Network Settings

LISTEN_HOST  = "0.0.0.0"      # Keep as-is to accept on all interfaces

LISTEN_PORT  = 8080            # Change if your firewall forwards a different port

RADIUS_HOST   = "172.16.0.12" # CHANGE: Set to your FortiGate IP

RADIUS_PORT   = 1813           # Default RADIUS accounting port

RADIUS_SECRET = "changeme"     # CHANGE: Set a strong shared secret

## 4.2  Log Directory

LOG_DIR = "/opt/mist-radius/logs"  # Use an absolute path

Log files are created automatically, one per day, named:

nac_accounting_YYYY-MM-DD.log

Each day’s file is created fresh or appended to if it already exists. The service handles log rotation internally — no external tool such as logrotate is required.

## 4.3  Optional Webhook Secret

If you configure a shared secret in the Mist webhook settings, set the same value here:

WEBHOOK_SECRET = "my-mist-webhook-secret"

Leave as None to disable signature verification.

# 5.  Mist Webhook Configuration

Log in to the Mist dashboard at manage.mist.com and navigate to:

Organisation → Settings → Webhooks → Add Webhook

Configure the webhook with the following values:

| Field | Value |
| --- | --- |
| Name | NAC Accounting Forwarder |
| URL | http://YOUR-SERVER-IP:8080 |
| Topics | nac-accounting |
| Secret | The value set in WEBHOOK_SECRET (optional) |
| Enabled | Yes |

Click Save. Mist will begin posting accounting events to your server whenever a client authenticates or disconnects.

# 6.  FortiGate RADIUS SSO Configuration

FortiGate’s RADIUS Single Sign-On (RSSO) feature listens for RADIUS Accounting packets and uses them to build a user-to-IP mapping table, which is then available to firewall policies.

## 6.1  Create the RSSO Entry

On the FortiGate, navigate to:

User & Authentication → RADIUS SSO → Create New

Configure as follows:

| Field | Value |
| --- | --- |
| Name | Mist-NAC |
| Primary Server IP | IP address of your forwarder server |
| Primary Server Port | 1813 |
| Secondary Server | (leave blank) |
| Secret | Must exactly match RADIUS_SECRET in script |
| RSSO Attribute | User-Name |
| RSSO Endpoint Attribute | Framed-IP-Address |
| RSSO Endpoint Block Attribute | (leave blank) |

## 6.2  Verify RSSO is Receiving Packets

After saving, trigger a client authentication event, then check the user identity table on the FortiGate CLI:

diagnose firewall auth list

A correctly-received session will show an entry such as:

user1@keeley.org.uk, 172.16.10.79, duration 120 secs

Additional RSSO diagnostics:

diagnose rsso status

diagnose rsso query username user1@keeley.org.uk

## 6.3  Use RSSO Identity in Firewall Policies

Once RSSO is receiving sessions, navigate to:

Policy & Objects → Firewall Policy → Create New

In the Source field, add the RSSO user group. This enables identity-based policies that allow or deny access based on authenticated username rather than IP address alone.

# 7.  Running as a System Service

Install the forwarder as a systemd service to ensure it starts automatically on boot and restarts on failure.

## 7.1  Create the Service File

sudo nano /etc/systemd/system/mist-radius.service

Paste the following content:

[Unit]

Description=Mist NAC to FortiGate RADIUS Accounting Forwarder

After=network.target

[Service]

Type=simple

User=nobody

WorkingDirectory=/opt/mist-radius

ExecStart=/usr/bin/python3 /opt/mist-radius/mist_nac_radius.py

Restart=on-failure

RestartSec=5

StandardOutput=journal

StandardError=journal

[Install]

WantedBy=multi-user.target

## 7.2  Enable and Start the Service

sudo systemctl daemon-reload

sudo systemctl enable mist-radius

sudo systemctl start mist-radius

## 7.3  Check Service Status

sudo systemctl status mist-radius

Expected output:

● mist-radius.service - Mist NAC to FortiGate RADIUS Accounting Forwarder

Loaded: loaded (/etc/systemd/system/mist-radius.service; enabled)

Active: active (running) since Mon 2026-05-18 12:00:00 UTC

Main PID: 1234 (python3)

## 7.4  View Live Logs

sudo journalctl -u mist-radius -f

# 8.  Testing and Verification

## 8.1  Expected Log Output

When the service starts:

2026-05-18 12:04:50  INFO  Webhook receiver  : 0.0.0.0:8080

2026-05-18 12:04:50  INFO  RADIUS Accounting : 172.16.0.12:1813

2026-05-18 12:04:50  INFO  Log directory     : /opt/mist-radius/logs

When a client connects:

2026-05-18 12:18:04  INFO  type=NAC_ACCOUNTING_START  user=user1@keeley.org.uk

mac=2a5bb814da3d  ip=-  session=0925DD44DCF80647

2026-05-18 12:18:04  INFO  RADIUS → 172.16.0.12:1813  type=NAC_ACCOUNTING_START

session=0925DD44DCF80647  (104 bytes)

## 8.2  Verify with Wireshark

Capture traffic on the forwarder server:

sudo tcpdump -i any -n udp port 1813 -w /tmp/radius_test.pcap

Open in Wireshark and apply the display filter: radius

Confirmed packet structure from a validated working capture:

| Attribute | Start | Update | Stop | Example Value |
| --- | --- | --- | --- | --- |
| Acct-Status-Type | ✓ | ✓ | ✓ | Start / Interim-Update / Stop |
| User-Name | ✓ | ✓ | ✓ | user1@keeley.org.uk |
| NAS-IP-Address | ✓ | ✓ | ✓ | 172.16.0.10 |
| Calling-Station-Id | ✓ | ✓ | ✓ | 2A-5B-B8-14-DA-3D |
| Acct-Session-Id | ✓ | ✓ | ✓ | 0925DD44DCF80647 |
| Called-Station-Id | ✓ | ✓ | ✓ | PK_NAC |
| Framed-IP-Address | – | ✓ | ✓ | 172.16.10.79 |
| Acct-Input-Octets | – | ✓ | ✓ | 212166 |
| Acct-Output-Octets | – | ✓ | ✓ | 210692 |
| Acct-Session-Time | – | – | ✓ | 120 (seconds) |
| Acct-Terminate-Cause | – | – | ✓ | User-Request |

Packet sizes from the confirmed capture: Start = 104 bytes, Interim-Update = 134 bytes, Stop = 146 bytes.

# 9.  Troubleshooting

## 9.1  Common Issues

Issue: “can’t concat str to bytes” error in logs

Cause: RADIUS_SECRET was set as a plain string. The current version of the script encodes the secret automatically, so this error should not occur. Ensure you are running the latest version.

Issue: No packets appearing in Wireshark on the FortiGate

- Confirm the forwarder can reach the FortiGate on UDP 1813:

nc -u -z -w2 172.16.0.12 1813 && echo reachable

- Check no local firewall is blocking outbound UDP 1813:

sudo iptables -L OUTPUT -n | grep 1813

- Confirm Mist webhook events are being received by checking the log file for incoming POST entries.

Issue: FortiGate RSSO not showing users

- Verify the shared secret matches exactly on both sides (case-sensitive).

- On FortiGate CLI run: diagnose rsso status

- Ensure the RSSO entry lists the correct server IP (the forwarder, not the Mist cloud).

- Check that the FortiGate interface policy allows inbound UDP 1813 from the forwarder.

Issue: Client IP missing on Start events

This is expected behaviour. Mist does not include client_ip in NAC_ACCOUNTING_START events. Framed-IP-Address is only present in UPDATE and STOP. FortiGate RSSO will complete the identity mapping when the first UPDATE is received.

Issue: Webhook port not reachable from Mist

- Ensure your edge firewall has a port-forward rule directing inbound TCP 8080 to this server.

- Allow the port through the local firewall:

sudo ufw allow 8080/tcp

## 9.2  Useful Commands

| Purpose | Command |
| --- | --- |
| Check service status | systemctl status mist-radius |
| Tail today’s log file | tail -f /opt/mist-radius/logs/nac_accounting_$(date +%Y-%m-%d).log |
| Test FortiGate UDP reachability | nc -u -z -w2 172.16.0.12 1813 |
| Check listener ports | ss -ulnp ¦ grep 1813 ; ss -tlnp ¦ grep 8080 |
| FortiGate identity table | diagnose firewall auth list |
| FortiGate RSSO status | diagnose rsso status |
| FortiGate RSSO query all | diagnose rsso query all |